# Technical information sheet for the proposed SDG indicator “International Internet bandwidth per inhabitant (bits/second/inhabitant)”

## Goal and target addressed:

17.8: fully operationalize the Technology Bank and STI (Science, Technology and Innovation) capacity building mechanism for LDCs by 2017, and enhance the use of enabling technologies, in particular Information and Communications Technology

# Definition and method of computation:

International Internet bandwidth refers to the total capacity of international telecommunication links provisioned to carry Internet traffic, in megabits per second (Mbit/s). If capacity is asymmetric (i.e. more incoming (downlink) than outgoing (uplink) capacity), then the incoming (downlink) capacity should be provided. Data on international Internet bandwidth refers to the used international Internet bandwidth (traffic) and to the average traffic load (expressed in Mbit/s) of international fibre-optic cables and radio links for carrying Internet traffic. The average should be calculated over the 12-month period of the reference year, and should take into consideration the traffic of all international Internet links. The combined average traffic load of different international Internet links can be reported as the addition of the average traffic load of each link.

The Internet is a worldwide public computer network. It provides access to a number of communication services including the World Wide Web and carries e-mail, news, entertainment and data files.

ITU collects data for this indicator through an annual questionnaire sent to national regulatory authorities or Information and Communication Technology Ministries, who collect the data from Internet Service Providers and/or wholesale Internet connectivity providers. The data can be collected by asking the operators that own the international fibre-optic cables and radio links about the bandwidth data from their links, excluding any leased capacity from third parties. Data can also be collected by asking retail Internet service providers about the international Internet bandwidth they use to carry the traffic from their users, irrespective of whether this capacity is owned or leased. Data are then added up to obtain the country totals.

This indicator refers to the used capacity of international connections between countries for transmitting Internet traffic. Out of the total international bandwidth available in the country (i.e. the potential capacity of the connections), there is a part that corresponds to the lit or equipped capacity, i.e. capacity currently available for use and for which the necessary equipment has been deployed and is operational at both ends of the link. Only the part of the lit/equipped capacity that has been actually used to carry Internet traffic during the reference period is counted as used capacity.

Data on international Internet bandwidth are multiplied by 1 million and divided by the population to derive the international Internet bandwidth per inhabitant (bits/second/inhabitant). Data are presented in relative terms to the population of the country in order to ascertain to what extent the current international Internet bandwidth suffices the whole population in a country.

# Rationale and interpretation

The Internet has become an increasingly important tool to provide access to information, and can help operationalize the Technology Bank and STI (Science, Technology and Innovation) capacity building mechanism, and enhance knowledge sharing. International Internet bandwidth is an important building block in providing high-speed Internet access and to ensure that Internet users have quality access to the global Internet and can take advantage of the growing amount of Internet content – including user-generated content –, services and information. Moreover, the lack of international Internet bandwidth has been a historical bottleneck in the broadband provision chain in developing countries, and therefore merits particular policy attention and monitoring.

Domestic and international backbones are important building blocks of Internet

infrastructure. Backbone transmission networks typically revolve around fibre-optic, satellite

and microwave infrastructure. Backbone transmission bandwidth affects the speed at which

information is delivered to, and sent from, Internet users. It is measured in the number of bits

that can be transferred per second. A common benchmark is bits per second per capita,

obtained by dividing the Internet bandwidth by the population.

Several countries in the world, and in particular LDCs, have only a very limited amount of international Internet bandwidth, which severely limits the potential of the Internet. The indicator also highlights the importance of Internet use as a development enabler and helps to measure the digital divide, which, if not properly addressed, will aggravate inequalities in all development domains. Information on these indicators will contribute to the design of targeted policies to overcome those divides.

# Sources and data collection

The ‘international Internet bandwidth per inhabitant’ indicator is based on an internationally agreed definition and methodology, which have been developed under the coordination of ITU, through its Expert Groups and following an extensive consultation process with countries. It is also a core indicator of the Partnership on Measuring ICT for Development's Core List of Indicators, which has been endorsed by the UN Statistical Commission (last time in 2014). Data on international Internet bandwidth are also used in the calculation of the ITU ICT Development Index (IDI), and thus considered a key metric for international comparisons of ICT developments.

ITU collects data for these indicators through an annual questionnaire sent to national regulatory authorities or Information and Communication Technology Ministries, who collect the data from Internet Service Providers and/or wholesale Internet connectivity providers. For countries that do not provide the information, ITU estimates the indicator based on information provided by operators/ISPs, and based on subscription data. By 2014, data are available for about 200 economies.

# Disaggregation

Since data for this indicator are based on administrative data from operators (not household surveys), data cannot be broken down by any individual characteristics.

# Comments and limitations

There exist different measurements of international Internet bandwidth, such as potential, lit/equipped, purchased and used capacity. The harmonization of the data reported into a single common metric remains a challenge. ITU is working towards the harmonization of the data on international Internet bandwidth through the work of the ITU Expert Group on Telecommunication/ICT Indicators (EGTI).

# Gender equality issues

Data cannot be broken down by gender.

# Data for global and regional monitoring

ITU produces regional and global aggregates of International Internet bandwidth per inhabitant, as well as for LDCs.

# Supplementary information

Year-end estimates are usually released in June of the following year through the ITU World Telecommunication/ICT Indicators Database.

# References:

- ​ITU Handbook for the collection of Administrative Data on Telecommunications/ICT, 2011 (and revisions and new indicators)

# Targets for which indicator are relevant:

9.a, 17.8