# River and lake water quality

### Goal and targets addressed

Goal 6: ensure availability and sustainable management of water and sanitation for all.

Target 6.3: By 2030, improve water quality by reducing pollution, eliminating dumping and minimizing release of hazardous chemicals and materials, halving the proportion of untreated wastewater and increasing recycling and safe reuse by [x] per cent globally.

### Indicators

Concentrations in nitrogen (phosphorus) and oxygen (in mg/l).

### Definition and method of computation

Indicators based on monitoring data and on the analysis of unfiltered samples, usually with a digestion step. Expressed as annual average concentrations in nitrogen (phosphorus) and oxygen (in mg/l), measured at the mouth or the downstream frontier of countries’ main rivers (i.e. those draining the large watersheds).

Usually computed as three-year averages of annual average concentrations, and showing trends over longer periods.

### Rationale and interpretation

Water quality (physical, chemical, microbial, biological) is of economic, environmental and social importance. It is affected by water abstraction, pollution loads from human activities, and by climate and weather.

If pressure from human activities becomes so intense that water quality is impaired to the point that it requires ever more advanced and costly treatment, or that aquatic plant and animal species in rivers and lakes are greatly reduced, then the sustainability of water resource use is in question.

The indicators show the evolution of water quality in rivers and lakes over longer periods (10 to 20 years). Measured at the mouth or downstream frontier of rivers they give a summary view of the pollution load and clean-up efforts in the upstream watershed..

### Sources and data collection

Source 1: OECD Environmental data.

Data collection from countries via questionnaire (OECD state of the environment questionnaire, section on inland waters)

Source 2: EEA Waterbase (for data on European countries).

### Disaggregation

By main rivers and by main lakes

### Comments and limitations

The calculation of the indicators requires the existence of water monitoring systems, and continuity in the monitoring effort.

### Gender equality issues

-

### Data for global and regional monitoring

OECD member and partner countries..

### References

OECD, "River and lake quality", http://dx.doi.org/10.1787/data-00693-en

# ODA for water supply and sanitation

### Goal and target addressed

Goal 6: Ensure availability and sustainable management of water and sanitation for all.

Target 6a: By 2030, expand international cooperation and capacity-building support to developing countries in water and sanitation related activities and programmes, including water harvesting, desalination, water efficiency, wastewater treatment, recycling and reuse technologies.

### Definition and method of computation

Total net official development assistance (ODA) to water supply and sanitation (purpose code 140).  Data expressed in US dollars at the average annual exchange rate.

### Rationale and interpretation

ODA is the accepted measure of international development co-operation.  In this case it captures aid in support of projects and programmes to improve water supply and sanitation infrastructure in developing countries.

### Sources and data collection

Data are compiled by the Development Assistance Committee (DAC) of the Organisation for Economic Co-operation and Development from returns submitted by its member countries and other aid providers.  Data are available here.

### Disaggregation

The data are generally obtained on an activity level, and include numerous parameters.  They can thus be disaggregated by provider and recipient country; by type of finance, and by type of resources provided.  Some data are also available on the policy objectives targeted by individual projects.

### Comments and limitations

The data only cover official concessional support from donor countries. The OECD and other organisations also collect data on broader investment flows to developing countries.  However detailed sectoral information on such flows is lacking.

### Gender equality issues

The data include a “gender equality” marker which identifies individual projects that have a clear gender dimension.

### Data for global and regional monitoring

Data are available for essentially all high-income countries, and for an increasing number of middle-income aid providers.

### Supplementary information

See Aid to water supply and sanitation

### References

OECD, 2015 Aid to the water supply and sanitation sector